| AWS to do list |  | Check after maintenance |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Part | Shipped (Y/N) | Replaced? (Y/N) | Number shipped/arrival | Number departure |  |
| Radiometer | Y | N |  | 090113 | Clean? Y / N |
| Ablation hose assembly | N |  |  | 5054 | No air in hose and bladder half full? Y / N: Unknown |
| Iridium modem IMEI number | N |  |  | 300234061852400 |  |
| Inclinometer/compass | N |  | Attached to the radiometer? Y / N |  |  |
| Satellite antenna | N |  |  |  |  |
| Wind sensor | N |  | Black box pointing toward the radiometer (i.e., along the boom)? Y / N |  |  |
| Temperature / humidity assembly | N |  | Fan spins? Y / N Sounds, OK? Y / N |  |  |
| HygroClip T/hum assembly | Y | Y | NB: Rotronics (Y / N) could have a 40 °C offset. New or old type? Square plug NB: Lufft sensor (Y / N) |  |  |
| Sonic ranger on AWS | Y |  | Old sensor with new membrane? Y / N |  |  |
| Sonic ranger on stake | Y |  | Old sensor with new membrane? Y / N NB: make sure not being buried next winter |  |  |
| Thermistor string | N |  |  |  |  |
| Solar panel | N |  | Clean? Y / N NB: is not effective when partially covered |  |  |
| GPS antenna | N |  | At top of logger enclosure? Y / N |  |  |
| Logger box ID | N |  | CR1000 or CR1000X? ID number: |  |  |
| Logger enclosure | N |  | Replaced (including everything inside)? Y / N New desiccant bags? Y / N |  |  |
| Battery box | N |  | New box? Y / N New desiccant bags? Y / N |  |  |
| Stakes | N |  | New one(s) drilled in? Y / N Length of new stake(s) 8 m? Y / N |  |  |
| Tripod | N |  | New tripod? Y / N New sensor boom? Y / N |  |  |

| Station name | Freya |
| --- | --- |
| People present | Signe HL, Anton, Bernard |
| Purpose | Maintenance / installation / take-down / quick visit / emergency |
| Date | 19 april + 28 april |

Maintenance sheet version 10 for ablation area (tripod, one level)

Notes:

| Station name | Freya |
| --- | --- |
| Date | 19 april + 28 april |

Metadata maintenance

|  | Arrival | Departure |
| --- | --- | --- |
| Time difference between logger clock and UTC (± mm:ss) | 30 sek | 30 sek |
| Download data and/or change CF/SD Card (wait for green light) | Card swapped | Y / N |
| Name of logger program | PROMICE_v2_2022_v1_1 | PROMICE_v2_2022_v1_1 |
| Screen dump of values in fast scan mode | Y / N | Y / N |
| Photos of tripod, sensors, logger box wiring etc. (more is better) | Y / N | Y / N |
| Boom direction relative to north | true/magn. | true/magn. |
| Box on wind sensor exactly towards mast? If not, measure. | Y / N | Y / N |
| Radiometer exactly aligned with inclinometer? If not, measure. | Y / N | Y / N |
| Temperature / humidity height (bottom of casing – surface) | cm | cm |
| Stake assembly standing? | Y / N / sort of | Y / N / sort of |
| Free length of stake | cm | cm |
| Height of ablation bladder fluid level above the ice surface | cm | cm |
| Vertical difference in surface height (at station – at hose) (approx.) | cm | cm |
| Length of thermistor string on surface from surface marking | cm | cm |
| Apparent damage to sensors? If yes, which? | Y / N | Y / N |

Notes:

Must-do at next visit:

Change the CNR4 (It is already at Zackenberg)

Redrill hose?