# MAINTAINCE SHEET FOR GC-NET v4

| Station name: | SDM |  | Date: | 03-05-2026 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| People present: | RSF, HETJA, IASTE, ARU |  | Maintenance |  | Install | Take-down | Data Download |

| Weather: |  |
| --- | --- |

| METADATA |  |  |
| --- | --- | --- |
|  | Arrival | Departure |
| Download Data | YES / NO | NA |
| Time difference between logger and UTC (± mm:ss) | 00:05 | 00:00 |
| Screenshot of ‘Status’ table | YES / NO | YES / NO |
| Screenshot of ‘Public’ table in FastScan mode | YES / NO | YES / NO |
| Photos of mast, loggerbox and sensors (inc. radiometer serial) | YES / NO | YES / NO |

| STATION POSITON AND OREINTATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Arrival |  | Departure |
| Lower vaisala height (see A) |  |  | 282 cm |  | 282 cm |
| Lower SR50 height (see A) |  |  | 262 cm |  | 262 cm |
| Upper vaisala height (see A) |  |  | 401 cm |  | 401 cm |
| Upper SR50 height (see A) |  |  | 387 cm |  | 387 cm |
| Upper boom direction (towards radiometer, see B) |  |  | 343 degree mag. |  | 343 degree mag. |
| Lower boom direction (towards lufft, see B) |  |  | 253 degree mag. |  | 253 degree mag. |
| Upper wind sensor orientation (away from black box, see C) |  |  | Yes |  | Yes |
| Lower wind sensor orientation (away from black box, see C) |  |  | Yes |  | Yes |
| Radiometer across-boom tilt, looking at the front (see D) |  |  | 2.5 degree to left/west |  | 2 degree to right/east |
| Mast tilt north-south axis |  |  | 1 degree down-south |  | 1 degree down-south |
| Mast tilt east-west axis |  |  | 2 degree to left/west |  | 2 degree left/west |
| Height of thermistor string pipe above surface |  |  | 60 cm |  | 265 cm |
| Depth of snowboard | 162 cm | Height of snowboard stake |  | 76 cm (bamboo stick) Vertical: YES / NO |  |

| Station name: | SDM |  | Date: | 03-05-2026 |
| --- | --- | --- | --- | --- |

| STATION HARDWARE |  |  |  |
| --- | --- | --- | --- |
| To do? | Done? | Item | Comments |
| Replaced | yes | Loggerbox | ID arrival: M4 - 06 ID departure: M4-10 |
| New | Yes | Modem IMEI | IMEI arrival: 300534064387800 IMEI departure:300534063818810 |
| Replace | Yes | Radiometer | Serial arrival: 110723 Serial departure: 213255 |
| Replace | Yes | Lufft | Done, cable on the Lufft end is held together by zip-ties. |
| Replace | Yes | Wind sensor upper | Type arrival: Black / White Type departure: Black / White |
| Replace | Yes | Wind sensor lower | Type arrival: Black / White Type departure: Black / White |
| Replace | Yes | Vaisala upper |  |
| Replace | Yes | Vaisala lower |  |
| Replace | Yes | SR50 upper |  |
| Replace | Yes | SR50 lower |  |
| Extend | Yes | Thermistor string |  |
|  |  | Iridium antenna | The cable end for the loggerbox was broken, the pin inside was gone. Replaced with a working cable, so everything works. |
|  |  | Solar panel |  |
| Replace | Yes | Battery box | Burial under lower lufft: YES / NO Location if NO: |
| Extend? | No | Mast |  |

NOTES (e.g. sensor damage, must-do at next visit)

- Lufft cable plug broken – change on next visit - Old Upper Vaisala reused - GNSS rover mounted 37cm above snow - New height of snowboard – pole: 148 cm and to bamboo: 242 cm - Lufft height 405 cm - No liquid GNSS installed. Antenna cable used for station Iridium antenna cable.