| Y |  | Check after maintenance |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Part | Shipped? (Y/N) | Replaced? (Y/N) | Number shipped/arrival | Number departure |  |
| Radiometer | N | N |  | 213254 | Clean? Y |
| Iridium modem IMEI number | Y | Y | Old:300534062419950 New: 300534063817770 | 300534063817770 |  |
| Inclinometer/compass | N | N | Attached to the radiometer? Y |  |  |
| Satellite antenna | N | N |  |  |  |
| Wind sensor upper | N | N | Black box pointing toward the radiometer (i.e., along the boom)? Y / N |  |  |
| Wind sensor lower | No, but we had one in stock | Y | Black box pointing same direction as the radiometer? Y / N |  |  |
| Temperature / humidity assembly upper | N | N | Fan spins? Y Sounds, OK? Y |  |  |
| Temperature / humidity assembly lower | Y | Y | Fan spins? Y Sounds, OK? Y |  |  |
| Sonic ranger upper | Y | y | Old sensor with new membrane? Y / N |  |  |
| Sonic ranger lower | Y | y | Old sensor with new membrane? Y / N |  |  |
| Thermistor string | Y | Y | Placed in a PVC tube |  |  |
| Solar panel | N | N | Clean? Y / N NB: is not effective when partially covered |  |  |
| GPS antenna | N | N | At top of logger enclosure? Y |  |  |
| Logger box ID |  |  | CR1000 or CR1000X? ID number: ? |  |  |
| Logger enclosure | Y | Y | Replaced (including everything inside)? Y / N New desiccant bags? Y / N |  |  |
| Battery box | N | N | New box? Y / N New desiccant bags? Y / N |  |  |
| Mast extension inserted? | Y | Y | How many inserted? How long? 1 extension of 1 m Notes: |  |  |
| Mast | N | N | New mast? Y / N New sensor boom? Y / N |  |  |

| Station name | Zac a |
| --- | --- |
| People present | Signe, berni, anton |
| Purpose | Maintenance / installation / take-down / quick visit / emergency |
| Date | 16 + 18 + 22 april 2025 |

Maintenance sheet version 10 for accumulation area (single mast, two levels)

Notes:

| Station name | ZAC_A |
| --- | --- |
| Date | 16 + 18 + 22 april 2025 |

Metadata maintenance

|  | Arrival | Departure |
| --- | --- | --- |
| Time difference between logger clock and UTC (± mm:ss) |  |  |
| Download data to PC and/or change SD card (wait for green light) | Y / N | Y / N |
| Name of logger program | GCNETv2_4 | GCNETv2_6 |
| Screen dump of values in fast scan mode | Y / N | Y / N |
| Photos of mast, sensors, logger box wiring etc. (more is better) | Y | Y / N |
| Upper boom direction with radiometer relative to north | magn. | 345 magn. |
| Lower boom perpendicular to upper boom. If not, measure. | Y | Y / N |
| Box on upper wind sensor exactly same direction as radiometer? If not, measure. | Y | Y / N |
| Box on lower wind sensor exactly same direction as radiometer? If not, measure. | broken | Y / N |
| Upper temperature / humidity height (bottom of casing – surface) | 255 cm | 305 cm |
| Lower temperature / humidity height (bottom of casing – surface) | 143 cm | 193 cm |
| Upper sonic ranger height (bottom of casing – surface) | 220 ca cm | 270 cm |
| Lower sonic ranger height (bottom of casing – surface) | 107 cm | 157 cm |
| Length of thermistor string on surface from surface marking | Unknown cm | * cm |
| Apparent damage to sensors? If yes, which? | Y lower wind | Y / N |

Notes:

* The last thermistor is 90 cm above the surface, but the string had to be folded to fit in the tube so the last thermistor is in fact only 70 cm above thesurface. This means that the thermistor is installed down to a depth of 930 cm

Must-do at next visit: