| AWS to do list |  | Check after maintenance |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Part | Shipped (Y/N) | Replaced? (Y/N) | Number shipped/arrival | Number departure |  |
| Radiometer | N | N |  | 131451 | Clean? Y |
| Ablation hose assembly | N | N | unknown | Leaking and not working | No air in hose and bladder half full? Y / N |
| Iridium modem IMEI number | N | N | 300534060520430 | 300534060520430 |  |
| Inclinometer/compass | N | N | Attached to the radiometer? Y |  |  |
| Satellite antenna | N | N |  |  |  |
| Wind sensor | N | N | Black box pointing toward the radiometer (i.e., along the boom)? Y |  |  |
| Temperature / humidity assembly | N | N | Fan spins? Y Sounds, OK? Y |  |  |
| HygroClip T/hum assembly | N | N | NB: Rotronics (Y / N) could have a 40 °C offset. New or old type? NB: Lufft sensor (Y / N) |  |  |
| Sonic ranger on AWS | y | Y | Old sensor with new membrane? |  |  |
| Sonic ranger on stake | y | Y | Old sensor with new membrane? NB: make sure not being buried next winter |  |  |
| Thermistor string | N | N |  |  |  |
| Solar panel | N | N | Clean? Y NB: is not effective when partially covered |  |  |
| GPS antenna | N | N | At top of logger enclosure? Y |  |  |
| Logger box ID | N |  | CR1000 or CR1000X? ID number: T3B-0002 |  |  |
| Logger enclosure | N | N | Replaced (including everything inside)? Y / N New desiccant bags? Y / N |  |  |
| Battery box | N | N | New box? N New desiccant bags? N |  |  |
| Stakes | N | N | New one(s) drilled in? Y Length of new stake(s) 8 m? Y |  |  |
| Tripod | N | N | New tripod? N New sensor boom? N |  |  |

| Station name | ZAC_L |
| --- | --- |
| People present | Signe, Anton, Bernhard |
| Purpose | Maintenance / installation / take-down / quick visit / emergency |
| Date | 16, 29 April 2025 |

Maintenance sheet version 10 for ablation area (tripod, one level)

Notes:

| Station name | Zac l |
| --- | --- |
| Date | 16, 29 april, 2025 |

Metadata maintenance

|  | Arrival | Departure |
| --- | --- | --- |
| Time difference between logger clock and UTC (± mm:ss) | 00:30 | 00:00 |
| Download data and/or change CF/SD Card (wait for green light) | Download Y / N | Y / N |
| Name of logger program | PROMICEv3_revired_1_4 | PROMICEv3_rewired_1_5 |
| Screen dump of values in fast scan mode | Y / N | Y / N |
| Photos of tripod, sensors, logger box wiring etc. (more is better) | Y / N | Y / N |
| Boom direction relative to north | 156 magn. | 156 true/magn. |
| Box on wind sensor exactly towards mast? If not, measure. | Y | Y |
| Radiometer exactly aligned with inclinometer? If not, measure. | Y / N build in | Y / N Build in |
| Temperature / humidity height (bottom of casing – surface) | 272 cm | 272 cm |
| Stake assembly standing? | Y | Y |
| Free length of stake | 378 cm | 140 cm |
| Height of ablation bladder fluid level above the ice surface | Not visible cm | Not visible (not working) cm |
| Vertical difference in surface height (at station – at hose) (approx.) | Flat cm | Flat cm |
| Length of thermistor string on surface from surface marking | Unknown cm | cm |
| Apparent damage to sensors? If yes, which? | Broken loggerbox Y | Y / N |

Notes:

The hose data cannot be trusted for 2024

Must-do at next visit:

Swap whole loggerbox because it has taken damage from being open and the connection with the USB does not seem to be stable.