AWS maintenance / establishment checklist

| Station name | TAS_A |
| --- | --- |
| People present | MCIT, MAV, Marek |
| Purpose of visit | Establishment of new AWS |
| Date & time | Arrival: 27.08.2013 12.30 Departure: 29.08.13 |
| Transport | By: Helicopter, Greenland Air From: Tasiilaq/TAS_L |
| Weather | Snowing |

Remarks on state of AWS / maintenance / establishment

Helicopter passed TAS_A and TAS_U on the way to TAS_L, where we unloaded equipment. 
Metadata upon arrival (Establishment of new AWS: Go to last page)

| Time difference between logger clock and UTC |  |
| --- | --- |
| Adjust logger time to UTC (be sure PC runs on UTC time) |  |
| Download data to PC and/or change CF Card (wait for green light) |  |
| Name of logger program |  |
| Photo or screen dump of values in fast scan mode |  |
| Latitude (dd mm.mmm) |  |
| Longitude (dd mm.mmm) |  |
| Altitude (m) |  |
| Photos of tripod, sensors, logger box wiring etc. (more is better) |  |
| Mast tilt in boom direction (+ if radiometer tilting down) |  |
| Mast tilt across boom looking from radiometer (- if clockwise) |  |
| Boom direction relative to north |  |
| Wind sensor box direction along boom? If not, measure direction. |  |
| Radiometer aligned with mast and inclinometer? If not, measure. |  |
| Temperature / humidity height (bottom of casing – surface) |  |
| Sonic ranger on AWS height (membrane – surface) |  |
| Wind sensor height (center of propeller – surface) |  |
| Sonic ranger on stakes height (membrane – surface) |  |
| Free length of stake – outer, holding boom with sonic ranger |  |
| Free length of stake – middle |  |
| Free length of stake – outer, holding boom without sonic ranger |  |
| Free length of ablation hose on/above the ice surface |  |
| Alt: depth of ablation hose using markings on hose |  |
| Height of ablation hose fluid level above the ice surface |  |
| Vertical difference in surface height (at station – at hose) (approx) |  |
| Length of thermistor string on surface from surface marking |  |
| Height of surface irregularities |  |
| Stake assembly still standing? |  |
| Apparent damage to sensors? If yes, which? |  |

Remarks:

Serial numbers and maintenance details

| Part | Old part number | New part number, if replaced | Potential maintenance tasks |
| --- | --- | --- | --- |
| Radiometer |  | 080024 | - Aligned with mast: N - Clean: Y |
| Inclinometer |  | 01355 | - Aligned with radiometer within 0.5: N (+/- 0.5o?) |
| Satellite antenna |  | GEUS34 |  |
| Wind sensor |  | 119282 |  |
| Temperature / humidity assembly (Fan, Bracket, MP100, Hygroclip) |  | ASS 10 2012 RS12T: 60981977 Bracket: 60998577 (DB) MP102: 60998574 H2-S3: 60956910 | - New HygroClip with old assembly: - Fan spins and sounds OK: Y Note Ass no. + serial no. for all 4 parts ! |
| Or: HygroClip only |  |  | - Large offset in HygroClip temperature: Y/N |
| Sonic ranger on AWS |  | 4110 | - Old sensor with new membrane: Y/N - Clicking as it should: Y |
| Solar panel |  | 9B1FC52 | - Output OK: Y - Clean: Y |
| GPS antenna |  | GEUS05 | - Stuck to top of logger enclosure: N |
| Data logger |  | 22106 WP E3200 | - New internal battery in old logger: N - New operating system: N - New logger program: 2012a.cr1 |
| Card reader |  | 8325 | - LED either green of flashing orange (active): Y |
| Multiplexer |  | E5918 | - Clicking as it should: Y |
| Iridium modem IMEI number |  | 1039970 | - Transmissions tested: N - Problems: Y/N |
| Barometer |  | 3696480 | ! changed to ! |
| Sonic ranger on stakes |  | 4284 | - Old sensor with new membrane: Y(?) - Clicking as it should: Y |
| Thermistor string |  | B04 | - New string in use, but old one still in ice: Y/N |
| Ablation hose assembly |  | 5013ABS | - New hose in use, but old one still in ice: Y/N - Air in current hose: Y/N - Air removed from current hose: Y/N - Added 50% antifreeze mix to bladder: Y/N |
| Logger enclosure | - Replaced (including everything inside): Y - New vent filter in logger enclosure: N - Moisture inside: N - New desiccant bags (2): Y |  |  |
| Battery box | - New box with new batteries: Y - Old box with new batteries: N - Moisture inside: N - New desiccant bags (2):Y |  |  |
| Stakes | - New ones drilled in: Y - Length: 8 m |  |  |
| Tripod | - New parts: Y |  |  |

- Metadata after maintenance / establishment

| Time difference between logger clock and UTC | 0 min |
| --- | --- |
| Adjust logger time to UTC (be sure PC runs on UTC time) | Y |
| Download data to PC and/or change CF Card (wait for green light) | Y |
| Name of logger program | Promice2013c.CR1 |
| Photo or screen dump of values in fast scan mode | Y |
| Latitude (dd mm.mmm) |  |
| Longitude (dd mm.mmm) |  |
| Altitude (m) |  |
| Photos of tripod, sensors, logger box wiring etc. (more is better) | Y |
| Mast tilt in boom direction (+ if radiometer tilting down) | +0.5 |
| Mast tilt across boom looking from radiometer (- if clockwise) | -1.5 |
| Boom direction relative to north (radiometer should point to true south) | 0 True |
| Wind sensor box direction along boom? If not, change. | Y |
| Radiometer aligned with mast and inclinometer? If not, align. | Y |
| Temperature / humidity height (bottom of casing – surface) | 265 cm |
| Sonic ranger on AWS height (membrane – surface) | 271 cm |
| Wind sensor height (center of propeller – surface) | 322 cm |
| Sonic ranger on stakes height (membrane – surface) | 35 cm |
| Free length of stake – outer, holding boom with sonic ranger | 52 cm |
| Free length of stake – middle | 72 cm |
| Free length of stake – outer, holding boom without sonic ranger | 64 cm |
| Free length of ablation hose on/above the ice surface | m |
| Alt: depth of ablation hose using markings on hose | cm |
| Height of ablation hose fluid level above the ice surface | 115 cm |
| Vertical difference in surface height (at station – at hose) (approx) | 20 cm |
| Length of thermistor string on surface from surface marking | -19 cm |
| Height of surface irregularities | 10 cm |

WS&D

Remarks:  MCIT drawing:

N

10o