AWS maintenance / establishment checklist

| Station name | TAS_L |
| --- | --- |
| People present | MCIT,MAV, Marek |
| Purpose of visit | Maintenance of AWS |
| Date & time | Arrival: 27.08.2013 12.30 Departure: 27.08.13 |
| Transport | By: Helicopter, Greenland Air From: |
| Weather | Overcast |

Remarks on state of AWS / maintenance / establishment

Helicopter passed TAS_A and TAS_U on the way to TAS_L, where we unloaded equipment. 
Metadata upon arrival (Establishment of new AWS: Go to last page)

| Time difference between logger clock and UTC | + 24 sec |
| --- | --- |
| Adjust logger time to UTC (be sure PC runs on UTC time) | Y/N |
| Download data to PC and/or change CF Card (wait for green light) | Both Y/N |
| Name of logger program | 2012a |
| Photo or screen dump of values in fast scan mode | Y/N |
| Latitude (dd mm.mmm) |  |
| Longitude (dd mm.mmm) |  |
| Altitude (m) |  |
| Photos of tripod, sensors, logger box wiring etc. (more is better) | Y/N |
| Mast tilt in boom direction (+ if radiometer tilting down) | +0.5o |
| Mast tilt across boom looking from radiometer (- if clockwise) | - 7.0o |
| Boom direction relative to north | ? |
| Wind sensor box direction along boom? If not, measure direction. | 6o off (SE?)Y/N |
| Radiometer aligned with mast and inclinometer? If not, measure. | Y/N |
| Temperature / humidity height (bottom of casing – surface) | 250 cm |
| Sonic ranger on AWS height (membrane – surface) | 258 cm |
| Wind sensor height (center of propeller – surface) | 306 cm |
| Sonic ranger on stakes height (membrane – surface) | 355 cm |
| Free length of stake – outer, holding boom with sonic ranger | 360 cm |
| Free length of stake – middle | 377 cm |
| Free length of stake – outer, holding boom without sonic ranger | 354 cm |
| Free length of ablation hose on/above the ice surface | 13.5 m |
| Alt: depth of ablation hose using markings on hose | N/A |
| Height of ablation hose fluid level above the ice surface | 139 cm |
| Vertical difference in surface height (at station – at hose) (approx) | 70 cm |
| Length of thermistor string on surface from surface marking | 340 cm |
| Height of surface irregularities | 30 cm |
| Stake assembly still standing? | Y/N |
| Apparent damage to sensors? If yes, which? | Y/N |

Remarks:

Interface 1 was missing internal wire between 8 and 9 and external wire between 4 and 10. Both were established externally.

Further remarks see the scanned maintenance sheet.
Serial numbers and maintenance details

| Part | Old part number | New part number, if replaced | Potential maintenance tasks |
| --- | --- | --- | --- |
| Radiometer | 080202 | “ | - Aligned with mast: N - Clean: Y |
| Inclinometer | 00634 | “ | - Aligned with radiometer within 0.5: N (+/- 0.5o?) |
| Satellite antenna | GEUS 11 | GEUS 28 |  |
| Wind sensor | 110445 | 83809 |  |
| Temperature / humidity assembly (Fan, Bracket, MP100, Hygroclip) | ASS: 8/2012 | RS12T: 611 38 896 Bracket: 611 54 942 MP102: 611 54 941 H2-S3: 611 14 636 | - New HygroClip with old assembly: - Fan spins and sounds OK: Y Note Ass no. + serial no. for all 4 parts ! |
| Or: HygroClip only |  |  | - Large offset in HygroClip temperature: Y/N |
| Sonic ranger on AWS | 3883 | “ | - Old sensor with new membrane: Y/N - Clicking as it should: Y |
| Solar panel |  | 4066283 (DB) | - Output OK: Y - Clean: Y |
| GPS antenna | None | GEUS04 | - Stuck to top of logger enclosure: N |
| Data logger | E3036 21576 | “ | - New internal battery in old logger: N - New operating system: N - New logger program: 2012a.cr1 |
| Card reader | 4592 | “ | - LED either green of flashing orange (active): Y |
| Multiplexer | E3334 | “ | - Clicking as it should: Y |
| Iridium modem IMEI number | None | 1627590 | - Transmissions tested: N - Problems: Y/N |
| Barometer | 3215015 | “ | ! changed to ! |
| Sonic ranger on stakes | No label | 1731 (DB) | - Old sensor with new membrane: Y(?) - Clicking as it should: Y |
| Thermistor string | ? | B08 (DB) | - New string in use, but old one still in ice: Y/N |
| Ablation hose assembly | 5018ABS | “ | - New hose in use, but old one still in ice: Y/N - Air in current hose: Y/N - Air removed from current hose: Y/N - Added 50% antifreeze mix to bladder: Y/N |
| Logger enclosure | - Replaced (including everything inside): N - New vent filter in logger enclosure: N - Moisture inside: N - New desiccant bags (2): Y |  |  |
| Battery box | - New box with new batteries: N - Old box with new batteries: N - Moisture inside: N - New desiccant bags (2):N, - not opened |  |  |
| Stakes | - New ones drilled in: Y - Length: 8 m |  |  |
| Tripod | - New parts: N |  |  |

- Metadata after maintenance / establishment

| Time difference between logger clock and UTC | 0 min |
| --- | --- |
| Adjust logger time to UTC (be sure PC runs on UTC time) | Y |
| Download data to PC and/or change CF Card (wait for green light) | Y |
| Name of logger program | Promice2013c.CR1 |
| Photo or screen dump of values in fast scan mode | Y |
| Latitude (dd mm.mmm) |  |
| Longitude (dd mm.mmm) |  |
| Altitude (m) |  |
| Photos of tripod, sensors, logger box wiring etc. (more is better) | Y |
| Mast tilt in boom direction (+ if radiometer tilting down) | -1 |
| Mast tilt across boom looking from radiometer (- if clockwise) | -4 |
| Boom direction relative to north (radiometer should point to true south) | 10 magn. (see drawing ) |
| Wind sensor box direction along boom? If not, change. | Y |
| Radiometer aligned with mast and inclinometer? If not, align. | Y |
| Temperature / humidity height (bottom of casing – surface) | 265 cm |
| Sonic ranger on AWS height (membrane – surface) | 280 cm |
| Wind sensor height (center of propeller – surface) | 322 cm |
| Sonic ranger on stakes height (membrane – surface) | 29 cm |
| Free length of stake – outer, holding boom with sonic ranger | 41 cm |
| Free length of stake – middle | 41 cm |
| Free length of stake – outer, holding boom without sonic ranger | 35 cm |
| Free length of ablation hose on/above the ice surface | m |
| Alt: depth of ablation hose using markings on hose | cm |
| Height of ablation hose fluid level above the ice surface | 133 cm |
| Vertical difference in surface height (at station – at hose) (approx) | -10 cm |
| Length of thermistor string on surface from surface marking | - |
| Height of surface irregularities | 20 cm |

WS&D

Remarks:  MCIT drawing:

N

10o